A short introduction to file marks:

===================================

File marks are written to tape to separate individual files (Note

that one `tar' file may contain many data files!). So several

files can be written to tape and individually retrieved.

The way ftape implements these file marks allows almost random

access to individual files (instead of sequentially reading the

entire tape until ones finds the right file mark).

Every time the device is closed after writing some data two file

marks are written to tape. If the non-rewinding device is used the

tape is rewound so it is positioned right between these two marks.

If another file is written to tape the second marks is overwritten

but the first stays causing succeeding files to be separated by

exactly one file mark.

The following should make this clear: After writing one file to the

non-rewinding tape device the tape layout will be:

<file-1> <eof> <eof>

When another file is written the layout will be:

<file 1> <eof> <file 2> <eof> <eof>

If we want to add another file to this tape after it has been

rewound, there are to options: First to skip 2 file marks to position

right between the last two marks. In this case we have to know how

many files to skip.

The other solution is to seek for the double file mark and position

in between. This is done with `mt eom'.

Remember to use the non-rewinding device when positioning the tape

or it will be rewound when closing !

Appending a file to a tape:

mt -f /dev/nftape eom

tar cvf /dev/ftape <files>

Reading the first and third files from tape:

mt -f /dev/ftape rewind

tar xvf /dev/nftape

mt -f /dev/nftape fsf 2

tar xvf /dev/nftape

The best way to understand what is happening is to picture the tape

layout and count the file marks that have to be skipped.

BTW: Keep in mind that file marks aren't written to tape until the

header segment is updated. For non-rewinding devices this will

not happen automatically: Make sure that either the last write is to

a rewinding device or issue an `mt rewind' or `mt rewoffl' to flush

the buffers. The last command will put the drive offline until you

reload the cartridge (or the driver). While the drive is offline all

commands will fail.