NAME

vgaset: a quick and dirty program to help setup X11 screen geometry.

SYNOPSIS

vgaset [-d<clock>] [-v] [-s settings]

DESCRIPTION

vgaset runs in an xterm and displays the current screen geometry in a

form similar to that used in a mode line in Xconfig. With single-key

commands, the geometry can be modified in real time, thus enabling

quick feedback for setting up optimal values for the hardware in use.

Optionally, vgaset will set initial values from the command line.

Command line options:

-v will cause vgaset to print out verbose information, in particular

the contents of the CRTC registers.

-d<clock> tells vgaset the speed of the dot clock being used. This

enables vgaset to print out timing information. <clock> may

be specified either in Hertz or in Megahertz.

-s<settings> gives the values of horizontal retrace start, horizontal

retrace end and horizontal total, vertical retrace start,

vertical retrace end and vertical total to set before

entering the command loop. These values are used in a

typical mode line:

"640x480"  28	  640 680 728 776 480 480 482 494

hrs hre ht      vrs vre vt

Commands:

l - decrease the left margin

L - increase the left margin

r - decrease the right margin

R - increase the right margin

h - decrease the horizontal sync pulse width

H - increase the horizontal sync pulse width

t - decrease the top margin

T - increase the top margin

b - decrease the bottom margin

B - increase the bottom margin

v - decrease the vertical sync pulse

V - increase the vertical sync pulse

i - turn interlace mode off (currently only Tseng 4000)

I - turn interlace mode on (currently only Tseng 4000)

? - reset initial values

= - modify register contents directly

CR - redisplay the settings. This can be useful after changing screen

mode with ctrl/alt/+ or ctrl/alt/-

q - quit

EOT - quit

After each command, vgaset will redisplay the current mode settings in

a form suitable for pasting into an Xconfig file - don't forget the

clock spec at the beginning of the line!

The `modify register' command prompts the user for the name of

a CRTC register and will then display the current value and

prompt for a new one. This can be used for more esoteric

modifications.

BUGS

Doubtless many. In particular, it's not quite clear whether

the description of the VGA registers I have is even moderately

accurate.

The restore initial values command will work OK if you have

broken things with the standard interactive commands, but it

doesn't restore all the registers, so you can really mess

things up with the explicit register set command. If this

happens, and you have a different resolution (under

X386/XFree86 only) you can probably save the day by changing

to a different resolution and then back again.

Chip recognition code is broken. This version assumes a Tseng

4000 chip. This is only relevant with the interlace mode

command.

The program has only been tested on a Diamond SpeedStar with an

ET4000 chip under BSD/386, and with an ET4000 clone under

System V.4.2. Possibly significant mods will be needed to get

it to work in other environments.

It is not possible to change the number or arrangement of

pixels displayed on the screen, since this would seriously

confuse the server.

AUTHOR

Greg Lehey, LEMIS, Schellnhausen 2, W-6324 Feldatal, Germany

Tel: +49-6637-1488, Fax: +49-6637-1489