15.2. Simulation Clocks

Each element in a simulation is associated with an interval timer or

``clock''.  This clock is used to determine how frequently the action

associated with the element (i.e., the INIT or PROCESS listed in the

simulation schedule) will be executed during the simulation process.

This is particularly useful when you want components of a simulation

to run at significantly different time scales.

By convention, clock number 0 is the global simulation clock or the

basic simulation time step.  All elements start out using clock 0 by

default.

The simulator also contains an array of 100 independent clocks (this

is the number of available clocks; a typical simulation only uses a

few of these clocks).  Each clock is identified by a number from 1 to

100 corresponding to its position in the clock array.  Clocks other

than clock 0 must have settings larger than that of clock 0; these

settings should be integer multiples of the clock 0 setting for

maximal timing precision.

The setting of a specific clock is not some ``current time'', but is

the time increment used for stepping its associated elements through

the simulation.  The units used for time must be consistent with those

used for other related variables and parameters of the simulation.

For example, if membrane resistances and capacitances are expressed in

kilohms and microfarads, then times should be expressed in

milliseconds.  To make it easier to keep track of units, many (but not

all) GENESIS simulations use SI (MKS) units, and express time in

seconds.

For example, suppose clock 0 has the setting 1.0 (its default

setting), and you assign clock 1 the value 5.  If you then have a

graphical element use clock 1 for its simulation time, the graph

element would perform its simulation action (e.g., receiving a message

and plotting a value) only once every 5 steps taken by  the other

elements.  This can greatly speed up a simulation, as the time

increment used for the display of information can usually be much

larger than that which is small enough for an accurate stepwise

numerical solution of the equations governing the model.

The following GENESIS routines are used for working with simulation

clocks:

-----------------------------------------------------------------------------

Routine       Description

-----------------------------------------------------------------------------

showclocks   Displays currently defined clocks and their assigned values.

useclock     Specifies which clock an element should use during simulation.

setclock     Sets time interval (step size) associated with specified clock.

getclock     Returns value of specified simulation clock.

-----------------------------------------------------------------------------

15.2.1.

Choosing a Simulation Time Step

At each step of a simulation, each participating element typically

performs one or more computations.  Some elements perform a stepwise

numerical integration in order to solve the state equations of the

element they represent.  When setting up a simulation, you will need

to choose a global simulation clock step size which is small enough to

give accurate results, yet is large enough to give a reasonable

simulation speed.

You can empirically determine the time step used with an integration

method by decreasing the time step until the differences in simulation

results are within some criterion.  As a starting point, you should

pick a step which would yield a smooth curve if you were to make a

``connect-the-dots'' type of plot of the most rapidly varying

variable.  The size of the time step needed also depends on the

integration method which is used.  The documentation for ``setmethod''

(setmethod.doc) describes the several different ways these integrals

can be calculated.

Typically, you need around 10 microseconds for a neural simulation

with explicit methods (like the default Exponential Euler method) and

50 to 100 microseconds with the implicit methods (Backward Euler and

Crank-Nicholson). (For the implicit methods the limiting factor is

often the speed of the channel kinetics.)  Implicit methods are

recommended when there are many small compartments in a cell model, as

these result in numerically ``stiff'' equations.

There is a complication when using the implicit methods. These are

inherently not object oriented, and involve the construction of an

``hsolve'' element which takes over the computations for a specified

cell.  (See the documentation for ``hsolve'' (hsolve.doc).)